The following describes the parallel port protocol as used by the PROMICE

attached to a CENTRONICS compatible parallel port:

Initialization:

PROMICE is reset by toggling the INIT line. If the PROMICE unit is

equipped with the Analysis Interface option (AI) and the AI was put

in transparant mode by using the AILOC command or similar thing then

the INIT line should be toggled about 4 times within a preiod of about

4 seconds to break the transparant mode link. The transparant mode link

may also be broken by sending a break-character if this mode of operation

was engaged.

Data transfer from HOST to the PROMICE:

This part of the protocol is simple. If BUSY is not asserted (generally

read as a '1' in the status register when BUSY is false) then the host

places the data on the bus and asserts STROBE and then deasserts it.

This would cause the PROMICE to assert BUSY ('0' in status register).

BUSY will remain asserted till PROMICE has read the parallel data. HOST

uses the BACK_ACK line to acknowledge the unasserted state of BUSY to the

PROMICE. The reason for this extra handshake is as follows:

Generally this much of the protocol is sufficient for one way data transfer.

However, the parallel port may be used in bidirectional data transfer. In

this case the PROMICE sends data back on the ERROR and STATUS lines by

putting 4-bits of data on them at a time. BUSY line is used to indicate

that PROMICE has put data on the ERROR/STATUS lines for the HOST to read.

This creates the problem that when the HOST samples the BUSY line and it is

asserted, then the HOST does not know for sure whether PROMICE has not

read the data sent to it previously or whether PROMICE is trying to send

data to the HOST.

To overcome this problem, the transfer protocol is further augmented by

the host acknowledging the false state of busy for each transfer. This

will become apparant when we examine the code.

Data transfer from PROMICE to the HOST:

PROMICE puts the first (low) nibble of data on the error lines as follows:

interface lines:	ACK		PE		SELECT		ERROR

data lines:	(lo)	D3		D2		D1			D0

data lines: (hi)	D7		D6		D5			D4

Then BUSY is asserted. Host reads the ERROR/STATUS lines and uses BACK_ACK

to indicate to the PROMICE that it has read the data. PROMICE then lowers

BUSY and the HOST acknowledges that by asserting BACK-ACK again. The process

repeats for the second nibble of the data.

MICRO-CODE version 6 enhancements:

This protocol was further modified by considering the case that the PROMICE

will not send data back until the entire command has been transfered from the

HOST to the PROMICE. And even then only if the command requires a response

from the PROMICE that the lines need to be turned around.  This fact is

used to substantailly improve the down-load speed of the parallel port.

The PICORE module's routine PCINIT automatically indicates to the PROMICE

to use the faster mode of transfer when it detects the micro-code version

6 and if the user has not explicitly over-ridden the use of fast transfer.

LoadICE will automatically engange in using the protocol in fast mode unless

the user explicitly wants to use the slow mode by specifying '-P' option on

the command line.

DAISY-CHAIN OF TWO PROMICE UNITS FOR 32 BIT EMULATION:

When two PROMICE units are daisy chained on the parallel port, the serial

daisy chain must also be used. Parallel port is used only for down-loading

the code. Serial port is used for control and read oprtaions.

The first unit with ID 0 and ID 1 (a duplex unit is assumed) is controlled

by using the BUSY and the STROBE lines. BUSY in its asserted state appears

as '0' in the status register.

The second unit with ID 2 and ID 3 is controlled by using PAPER and AUTOON

lines.  PAPER in its asserted state appears as '1' in the status register.

PARALLEL PORT LOGIC:

This is the actual driver used to implement the parallel port support on a

SUN workstation by using a SBUS adpater board. This may be modified for

support of other parallel port devices on UNIX systems. The sources to this

driver are called PPGEI.C and PPREG.H. The makefile is called MAKEFILE.PPG

and the command to load the driver is called MODLOAD.CMD

The driver may be compiled with the PPDBUG defined to turn on print statements

in the driver.

For PROMICE units with micro-code version 5 and below, the value of PIDLY must

be set to 5 or more.

The logic for transfer is primarily contained in the 'ppstrategy' routine:

/*

SBus Centronic Parallel Controller Driver

Version 1.0

Copyright (C) 1993  Grammar Engine, Inc.

*/

#ident "%Z%%M% %I% %E% Grammar Engine, Inc."

#include "ppreg.h"			/* register definitions */

#include <sun/vddrv.h>

#include <sys/ioctl.h>

#include <sys/param.h>

#include <sys/syslog.h>

#include <sys/buf.h>

#include <sys/uio.h>

#include <sys/errno.h>

#include <sys/file.h>

#include <sys/conf.h>

#include <sundev/mbvar.h>

#include <machine/psl.h>

#include <sun/openprom.h>

#define LOADABLE 1	/* for use with 'modload' command */

#define	PIDLY	1	/* micro seconds between pp I/O */

/*

* Buffers for use by physio().

*/

struct buf ppbuf[3];

#define	PPBUFSIZ	260

#define MAXPPS		3

/*

* software state structure, one for each printer

*/

struct pp_unit {

int		pp_flags;	/* printer state: */

#define	PP_OPEN		0x01		/* currently open */

#define	PP_TOUT		0x02		/* timeout on some operation */

#define	PP_TURBO	0x04		/* TURBO mode down-load */

#define	PP_TIMER	0x08		/* watchdog timer is running */

#define	PP_BUSY		0x10		/*	i/o in progress */

char		pp_timer;	/* for detecting timeout situations*/

struct dev_info *devinfo_p;	/* pointer to dev_info */

struct buf 	*pp_bp;		/* pointer to current 'buf' */

char		pp_buf[PPBUFSIZ]; /* buffer */

char		*pp_cp;	/* current byte in current buffer */

int		pp_count;	/* number of bytes left to print */

struct pp_reg	*pp_regbase;	/* device register base in Sbus space */

};

#define	PPREG_DATA	(pp->pp_regbase->pp_data)

#define	PPREG_DATA_IN	(pp->pp_regbase->pp_data_in)

#define	PPREG_CTRL	(pp->pp_regbase->pp_cntrl)

#define	PPREG_STAT	(pp->pp_regbase->pp_stat)

#define	BUSY	0x80

#define	STRON	0x05

#define	STROFF	0x04

#define	B_ACK	0x0C

#define	PAPER	0x20

#define	AUTOON	0x06

#define	AUTOOFF	0x04

#define	PPUNIT(dev)	(minor(dev) & 0xf)

#define	PPDIAG(dev)	(minor(dev) & 0x10)

extern  int hz;

#define	PPTICKS		7

int ppidentify(), ppattach(), ppintr(), pptimeout(), pppoll();

int ppopen(), ppclose(), ppread(), ppwrite(), ppioctl();

void show_devinfo(),ppdismiss();

struct dev_ops pp_ops = {

1,

ppidentify,

ppattach,

};

#if	defined(LOADABLE)

extern int nulldev();

extern int seltrue();

struct cdevsw pp_cdevsw = {

ppopen, ppclose, ppread, ppwrite, ppioctl, nulldev, seltrue, 0, 0

};

struct vdldrv pp_drv = {

VDMAGIC_DRV,			/* Drv_magic	*/

"PROMICE pp",			/* Drv_name	*/

&pp_ops,			/* Drv_dev_ops */

NULL,				/* Drv_bdevsw	*/

&pp_cdevsw,			/* Drv_cdevsw	*/

0,				/* Drv_blockmajor */

0				/* Drv_charmajor */

};

#endif

static u_char npps=0;

static struct pp_unit *pp_unit_array 	= NULL;

static u_char logpri = 0;

ppidentify(name)

char *name;

{

if(strcmp(name,"ELTK,pp") == 0)

{

if(npps < MAXPPS)

{

npps++;

return(1);    /* match one again */

}

else

uprintf("pp_identify: too many pps (%d).\n",npps);

}

return(0); 				/* no match or over limit */

}

ppattach(devinfo_p)

register struct dev_info *devinfo_p;

{

register struct pp_unit *pp;

struct dev_reg *dev_reg_p;

struct dev_intr *dev_intr_p;

u_int 	i,unit_no;

u_int 	interrupt_pri=0;

static int unit_count = 0;

register int s;

if(pp_unit_array == NULL)

{

pp_unit_array=(struct pp_unit *)

kmem_zalloc(npps * sizeof (struct pp_unit));

}

unit_no = unit_count++;

devinfo_p->devi_unit = unit_no;

pp_unit_array[unit_no].devinfo_p = devinfo_p;

(void) show_devinfo(devinfo_p);

pp=&pp_unit_array[unit_no];

dev_reg_p = devinfo_p->devi_reg;

pp_unit_array[unit_no].pp_regbase =  (struct pp_reg *)

map_regs(dev_reg_p->reg_addr,sizeof(struct pp_reg),

dev_reg_p->reg_bustype);

if(pp_unit_array[unit_no].pp_regbase == NULL)

{

uprintf("pp_attach: map_regs failed!\n");

return(-1);

}

report_dev(devinfo_p);

PPREG_CTRL = PC_OFF;

return(0);

}

ppopen(dev, flags)

dev_t dev;

int flags;

{

register struct pp_unit *pp = &pp_unit_array[PPUNIT(dev)];

u_char	unit_no;

unit_no = PPUNIT(dev);

if (unit_no >= npps)

return(ENXIO);

if (pp->pp_flags & PP_OPEN) 	/* enforce exclusive access */

return(EBUSY);

PPREG_CTRL = PC_OFF;		/* disable interrupts */

DELAY(500); /* adjust */

if ((pp->pp_flags & PP_TIMER)  ==  0) /*watchdog timer not running*/

{

/*

* Kick off watchdog timer.

*/

timeout(pptimeout, (caddr_t)pp, hz);

pp->pp_timer = 0;

pp->pp_flags |= PP_TIMER;

}

pp->pp_flags |= PP_OPEN;

#ifdef	PPDBUG

uprintf("\nPi_OPEN ");

#endif

return(0);

}

/*

* ppclose:

*	Close the printer device.

*/

ppclose(dev)

dev_t dev;

{

register struct pp_unit *pp = &pp_unit_array[PPUNIT(dev)];

untimeout(pptimeout, (caddr_t)pp);

pp->pp_flags = 0;

#ifdef	PPDBUG

uprintf("\nPi_CLOSE ");

#endif

}

ppread(dev, uio)

dev_t dev;

struct uio *uio;

{

int ppstrategy();

void ppminphys();

return(physio(ppstrategy, &ppbuf[PPUNIT(dev)], dev, B_READ,

ppminphys, uio));

}

ppwrite(dev, uio)

dev_t dev;

struct uio *uio;

{

int ppstrategy();

void ppminphys();

return(physio(ppstrategy, &ppbuf[PPUNIT(dev)], dev, B_WRITE,

ppminphys, uio));

}

/*

* ppstrategy:

*/

ppstrategy(bp)

register struct buf *bp;

{

register struct pp_unit *pp = &pp_unit_array[PPUNIT(bp->b_dev)];

int s;

int len,i;

unsigned char c,uid;

pp->pp_bp = bp;

pp->pp_count = bp->b_bcount;

pp->pp_cp = pp->pp_buf;

/* first determine if its a read or a write */

if (bp->b_flags&B_READ)	/* read from PROMICE */

{

#ifdef	PPDBUG

uprintf("\nPi_READ ");

#endif

pp->pp_flags |= PP_BUSY;

pp->pp_flags &= ~PP_TOUT;

pp->pp_timer = PPTICKS;		/* set timer */

/*	if in fast transfer mode then complete the back-ack handshake of the

previous 'write' command */

if (pp->pp_flags&PP_TURBO)

{

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK;

DELAY(PIDLY);

PPREG_CTRL = STROFF;

DELAY(PIDLY);

}

/* any response is at least 4 bytes long, so read four for sure */

for (i=0; i<4; i++)

{

while(PPREG_STAT&BUSY && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

/* low nibble */

c = (char)((PPREG_STAT>>3)&0x0f); DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(PPREG_STAT&BUSY && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

/* high nibble */

c |= (char)((PPREG_STAT<<1)&0xf0); DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

*pp->pp_cp = c;

pp->pp_cp++;

#ifdef	PPDBUG

uprintf("%2x< ",c);

#endif

}

if (pp->pp_flags&PP_TOUT)

{

bp->b_flags |= B_ERROR;

bp->b_error = EFAULT;

ppiodone(pp);

return;

}

/* now see if more stuff should be read from the PROMICE */

pp->pp_count = 4;

len = (int)(*(pp->pp_buf+2));

if (!len)

len = 256;

len += 3;

for (i=4; i<len; i++)

{

while(PPREG_STAT&BUSY && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

c = (char)((PPREG_STAT>>3)&0x0f); DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(PPREG_STAT&BUSY && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

c |= (char)((PPREG_STAT<<1)&0xf0); DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK; DELAY(PIDLY);

PPREG_CTRL = STROFF; DELAY(PIDLY);

*pp->pp_cp = c;

pp->pp_cp++;

pp->pp_count++;

#ifdef	PPDBUG

uprintf("%2x< ",c);

#endif

}

bp->b_bcount = pp->pp_count;

if(copyout(pp->pp_buf, bp->b_un.b_addr, (u_int) bp->b_bcount))

{

bp->b_flags |= B_ERROR;

bp->b_error = EFAULT;

ppiodone(pp);

return;

}

}

else	/* write to the PROMICE */

{

if (copyin(bp->b_un.b_addr, pp->pp_buf, (u_int) bp->b_bcount))

{

bp->b_flags |= B_ERROR;

bp->b_error = EFAULT;

ppiodone(pp);

return;

}

pp->pp_flags |= PP_BUSY;

pp->pp_flags &= ~PP_TOUT;

pp->pp_timer = PPTICKS;		/* set timer */

uid = *pp->pp_buf;

#ifdef	PPDBUG

uprintf("\nPi_WRITE(%d:%d) ",uid,pp->pp_count);

#endif

while (pp->pp_count && !(pp->pp_flags&PP_TOUT))

{

/* for two PROMICEs daisy chained on the parallel port, the ID determines

what lines we will use for down-loading the data. For ID 0 and 1 we use

the standard line and for ID 2 and 3 the other lines */

if (uid<2)

{

#ifdef	PPDBUG

uprintf("%2x",*pp->pp_cp);

#endif

while(!(PPREG_STAT&BUSY) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_DATA = *pp->pp_cp;

DELAY(PIDLY);

PPREG_CTRL = STRON;

DELAY(PIDLY);

PPREG_CTRL = STROFF;

DELAY(PIDLY);

/* if not in fast mode then do the back-ack handshake */

if (!(pp->pp_flags&PP_TURBO))

{

while(!(PPREG_STAT&BUSY) &&

!(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK;

DELAY(PIDLY);

PPREG_CTRL = STROFF;

DELAY(PIDLY);

}

#ifdef	PPDBUG

uprintf("> ");

#endif

}

else

{

/*	for the second unit use different lines for handshake */

#ifdef	PPDBUG

uprintf("%2x",*pp->pp_cp);

#endif

while((PPREG_STAT&PAPER) && !(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_DATA = *pp->pp_cp;

DELAY(PIDLY);

PPREG_CTRL = AUTOON;

DELAY(PIDLY);

PPREG_CTRL = AUTOOFF;

DELAY(PIDLY);

if (!(pp->pp_flags&PP_TURBO))

{

while((PPREG_STAT&PAPER) &&

!(pp->pp_flags&PP_TOUT))

DELAY(PIDLY);

PPREG_CTRL = B_ACK;

DELAY(PIDLY);

PPREG_CTRL = AUTOOFF;

DELAY(PIDLY);

}

#ifdef	PPDBUG

uprintf("* ");

#endif

}

pp->pp_cp++;

pp->pp_count--;

}

}

pp->pp_timer = 0;		/* turn off timer */

if (pp->pp_flags&PP_TOUT)

{

bp->b_flags |= B_ERROR;

bp->b_error = EFAULT;

ppiodone(pp);

return;

}

ppiodone(pp);

}

void

ppminphys(bp)

register struct buf *bp;

{

if (bp->b_bcount > PPBUFSIZ)

bp->b_bcount = PPBUFSIZ;

}

/*

* ppintr:

*	PROMICE does not cause any interrupts

*/

ppintr(unit_no)

u_int unit_no;

{

return(1);

}

/*

* pptimeout:

*	Check occasionally for lost interrupts or

*	printer errors (no paper, printer off line, etc.).

*/

pptimeout(arg)

caddr_t arg;

{

register struct pp_unit *pp = (struct pp_unit *)arg;

if ((pp->pp_flags & PP_OPEN)  ==  0) /* dev not open */

{

return;

}

else

if (pp->pp_timer <= 0) /* timer not currently active */

{

timeout(pptimeout, (caddr_t)pp, hz);

return;

}

if (--pp->pp_timer == 0)

{

/*

* Timer has expired - see what's wrong.

*/

#ifdef	PPDBUG

uprintf("\nPi_TOUT ");

#endif

pp->pp_flags |= PP_TOUT;

}

timeout(pptimeout, (caddr_t)pp, hz);

}

/*ARGSUSED*/

ppioctl(dev, cmd, data, flag)

dev_t dev;

int cmd;

caddr_t data;

int flag;

{

register struct pp_unit *pp = &pp_unit_array[PPUNIT(dev)];

unsigned char *udata;

udata = (unsigned char *) data;

switch(cmd)

{

case PPIOCGETS:

*udata = PPREG_STAT;

break;

case PPIOCGETC:

*udata = PPREG_CTRL;

break;

case PPIOCSETC:

PPREG_CTRL = *udata;

break;

case PPIOCGETD:

*udata = PPREG_DATA_IN;

break;

case PPIOCSETD:

PPREG_DATA= *udata;

break;

/* this was added so that LoadICE can set fast transfer mode */

case PPIOCSETF:

pp->pp_flags |= PP_TURBO;

break;

default:

return(ENOTTY);

}

return(0);

}

/*

* ppiodone:

*	Private version of 'biodone()'.

*/

ppiodone(pp)

register struct pp_unit *pp;

{

register struct buf *bp = pp->pp_bp;

bp->b_flags |= B_DONE;

pp->pp_flags &= ~PP_BUSY;

#ifdef	PPDBUG

uprintf("\nPi_IODONE");

#endif

}

/*

* Handle a polling Parallel Printer interrupt.

*/

pppoll()

{

return(1);

}

static void

show_devinfo(devinfo_p)

register struct dev_info	*devinfo_p;

{

struct dev_reg 	*dev_reg_p;

struct dev_intr	*dev_intr_p;

u_int	counter;

dev_reg_p = devinfo_p->devi_reg;

for(counter=1; counter <= devinfo_p->devi_nreg; ++counter,++dev_reg_p){

if(dev_reg_p == NULL) break;

}

dev_intr_p = devinfo_p -> devi_intr;

for(counter=1; counter <= devinfo_p->devi_nintr; ++counter,++dev_intr_p){

if(dev_intr_p == NULL) break;

}

return;

}

#if 	defined(LOADABLE)

int ppvdcmd(command, vdp, vdi, vds)

int			command;

struct	vddrv		*vdp;

struct	vdioctl_load	*vdi;

struct	vdstat		*vds;

{

struct	vdconf		*vdconf_p;

char			*string_p;

u_int			pp_unit_no;

int			status = 0;

switch(command){

case VDLOAD:

if(vdi != NULL){

if(vdi->vdi_userconf != NULL) {

vdconf_p = vdi->vdi_userconf;

if(vdconf_p->vdc_type != VDCEND) {

uprintf("........OK.\n");

}

while(vdconf_p->vdc_type != VDCEND){

switch(vdconf_p->vdc_type){

case VDCCONTROLLER:

string_p="(VDCCONTROLLER)";

break;

case VDCDEVICE:

string_p="(VDCDEVICE)";

break;

case VDCBLOCKMAJOR:

string_p="(VDCBLOCKMAJOR)";

break;

case VDCCHARMAJOR:

string_p="(VDCCHARMAJOR)";

break;

case VDCSYSCALLNUM:

string_p="(VDCSYSCALLNUM)";

break;

default:

string_p="(SWITCH ERROR!)";

break;

}

vdconf_p++;

}   /* while */

}

}

vdp->vdd_vdtab = (struct vdlinkage *)&pp_drv;

break;

case VDUNLOAD:

for(pp_unit_no =0;pp_unit_no < npps;pp_unit_no++){

if(pp_unit_array[pp_unit_no].pp_flags != 0)

status = EBUSY;

}

if(status == 0) {

(void) ppdismiss();

}

break;

case VDSTAT:

break;

default:

status=EINVAL;

break;

}

return(status);

}

static void

ppdismiss()

{

}

#endif