Software diskette #2

-----------------------------

1.Programmer Files :

PALP1.EXE : 1st generation PAL programmer software.

16R8 series,20R8 series...

PALP2.EXE : 1st generation PAL with polarity fuse.

16RA8 series,20RA10 series...

PALP3.EXE : new generation EPLD(macro cell output) include

UV erasable PAL(20G10,22V10),

Electrial erasable GAL(16V8,20V8,16V8A,20V8A),

or PEEL(18CV8,22CV10)....(Use GAL1.EXE)

NOTE:

During programming ICT PEEL 18CV8,22CV10, please

add 0.01 UF cap between pin 30 and pin 20.(Use PEEL1.EXE)

PALP4.EXE : new generation EPLD(macro cell output) include

UV erasable TIBPAL22V10/A/-15,22VP10/-20/-25,

TICPAL18V8,

SIGNETICS PLC16V8,PLC20V8,

PLC18V8Z,PLC22V8Z.

PALP5.EXE : AMD22V10-15/20,ATMEL22V10/10L are added,

user has to add a capacitor 0.01 uF(or greater) between

pin 32 and pin 20.

2.REVISION of programmer software :

Hardware Revision on :

VERSION 1 : 1st version hardware.4/15/1990.

current limit resistor from 22 ohm

to 10 ohm.

VERSION 2 : 2nd version hardware.12/1/1990.

Add GND control for PIN 30 to solve

40 PIN word-wide EPROM problom.

PALP1.EXE : V1.00 : 1st released version.

V1.01 : clarify type of TIBPAL.

V1.02 : increase 20L8B-2/D,20R8B-2/D,20R6B-2/D,20R4B-2/D.

JEDEC file saving error on 10H8,10L8 are solved.

V1.03 : add old 16L8-12/-15/-25 series security fuse programming

(21 volt).

new 16L8-12/-15/-25 series security fuse programming

voltage is 14 volts.

V1.04 : Add SIGNETICS PLUS16L8/R4/R6/R8,PLUS20L8/R4/R6/R8

V2.00 : TIBPAL 16L8-7/H-15,R4-7/H-15,R6-7/H-15,R8-7/H-15 included

V3.00 : new version.

V3.14 : 1/1/91.

V3.16 : 2/8/91, Add TI TIBPAD16N8-7,TIBPAD18N8-6

TIBPAL16xx-5. xx=L8,R4,R6,R8.

TIBPAL20xx-5. xx=L8,R4,R6,R8.

Improve AMD PAL16L8-5/-7/H-10 programming ALGO.

Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.17 : 4/1/91, Update load file.

V3.18 : 6/28/91,Revision of programming voltage for TI Mfr.

V3.19 : 7/1/91, Revision of programming algorithm for

TI 20l8-15/25, MMI 16l8D/B.

V3.20 : 8/27/91,Revision of text on the screen.

Revision of programming algorithm for NS PAL.

V3.24 : 1/15/92,Revision of TIBPAL20R4/R6/R8/L8-5,TIBPAL16N8-7

programming voltage.

PALP2.EXE : V3.00 : new version.

V3.11 : 1/1/91.

V3.12 : 2/8/91, Correct the problem that the characters(P and

H) can't be typed while executes "LOAD File"

function.

V3.13 : 4/1/91, Update load file.

V3.15 : 7/1/91, Add SEEQ PQ20RA10Z-45.

V3.16 : 8/27/91,Revision of programming algorithm for NS 20RA10.

V3.19 : 1/15/92,Add RICOH EPL16P8B.

PALP3.EXE : V1.00 : 1st released version.

V1.01 : solve AUTO function bug.

V1.02 : solve GAL 16V8,20V8 JEDEC file loading error at UES,PTD

V1.03 : solve GAL 16V8,20V8 programming at 1's and 0's.

We made a mistake on GAL JEDEC file in 1's and 0's

definition,the fuses were programmed to it's inverted

status at V1.02 or less version.

We are very appreciated that MICROWARE,EAGLE,BLISS,BALTEC

payed their attention and help us to solve these

problems.

V1.04 : ICT and HYUNDAI PEEL 18CV8 are available now.

V1.05 : ICT PEEL 22CV10 are available now.

V1.06 : SECURITY VOLTAGE OF ICT 18CV8 down to 15.0 VOLTS.

ADD TICPAL22V10.

ADD SAMSUNG 22V10.

ADD VLSI,SGS GAL16V8 and GAL20V8.

ADD AMI,GOULD PEEL18CV8.

V1.07 : A VERSION GAL(16V8A,20V8A) ARE AVAILABLE.

V3.00 : new version.

V3.13 : 1/1/91.

V3.14 : 2/8/91, Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.15 : 4/1/91, Update load file.

V3.16 : 7/1/91, Add CYPRESS PLDC20G10/A,PALC22V10/A,PLDC20G10B,

PALC22V10B.

V3.17 : 8/16/91,Revision of programming voltage for

CYPRESS PLDC20G10B,PALC22V10B.

V3.18 : 10/1/91,Add CYPRESS PLD20RA10.

V3.20 : 1/15/92.

PALP4.EXE : V1.00 : 1st released version.

V3.00 : new version.

V3.02 : 1/1/91.

V3.03 : 5/1/91.

V3.04 : 7/1/91, Revision of text on the screen.

V3.08 : 1/15/92.

PALP5.EXE : V1.00 : 1st released version

AMD22V10-15/20,ATMEL22V10/10L are added, but user has to

add a capacitor 0.01 uF(or greater) between pin 32

and pin 20.

V3.00 : new version.

V3.03 : 1/1/91.

V3.04 : 2/8/91, Add TI TIBFPLS506,TIBPSG507

Correct the problem that the characters(P and H

) can't be typed while executes "LOAD File"

function.

V3.05 : 4/1/91, Update load file.

V3.06 : 7/1/91, Revision of programming voltage for TI Mfr.

V3.07 : 8/16/91,Revision of programming voltage for AMD 22V10.

V3.08 : 9/27/91,Add SIGNETICS PLV750.

V3.09 : 10/1/91,Eliminate errors occured in reading for

ATMEL AT22V10L.

V3.11 : 10/28/91,Revision delay time for AMD AmPAL22V10.

V3.13 : 1/15/92.

3.NOTE : We will keep updating software for new programmable devices.

Special device will be arranged to be 1st priority, if user offer us

the programming information and JEDEC file assignment of that device.

4.Powerful screen editor function description :

Some users said that it is unconvenient to specify the address by segment

prefix form during DEBUG MODE .(such as F000:E000 represent address FE000).

User only want to enter the offset address in memory buffer.

According to upper reasons, we develop a simple screen editor that will

meet the requirment of users.

Although the DEBUG.COM has been replaced, but user still can invoke it

by SHELL( DOT ) command in this editor.

Editor command are almost the same as DEBUG.COM, such as

D,E,M,F. It is not necessary to enter the segment address before

the start address,end address. Only the offset address in memory buffer is

needed.

The extra commands are shown below.

E : Same as E command in DEBUG.COM, except that user can modified

data in full screen editing mode.

There are two editing fields, BINary and ASCII, you can edit

any datas on each field.

P : Same as D command,except that the data will be printed to the

printer.

C : The command will calculate the total sum between start and end

address.

S : Will search the ASCII string that are included between start and

end address.

B : Will search the BINARY data that are included between start and

end address.

. : Will execute the excutable file and pass argument to the file.

This command liked the BASIC SHELL command can invoke any file and

pass controll to it. So you can invoke your own editor to do more

powerful job.

Note : The actual segment address of memory buffer shown on

main menu is for referenced only. You can pass this parameter

to the invoked file for special purpose.

? : Will display the editor commands.

------------------------ END -----------------------------